Top of Form

## Inputs

Power generation

W

Average power available from the solar array during sunlit time.

Orbit period

min

Orbital period of the spacecraft.

Eclipse duration

min

Time spent in eclipse per orbit.

Load power sunlit

W

Average total load power during sunlit period.

Load power eclipse

W

Average total load power during eclipse period.

Battery depth of discharge

%

Allowed depth of discharge as a percentage

## Outputs

Sunlit duration

55 min

Average load power

42.22 W

Energy generated per orbit

91.67 Wh

Energy consumed per orbit

63.33 Wh

Net energy per orbit

28.33 Wh

Power margin

44.74 %

Required battery energy

17.5 Wh

Required battery capacity

87.5 Wh

Bottom of Form

## Methodology

### Time breakdown

The sunlit duration is calculated as:

t sunlit = t orbit − t eclipse

where:

- tsunlit = sunlit duration (min)

- torbit = orbit period (min)

- teclipse = eclipse duration (min)

### Energy generation

Energy generated per orbit is calculated as:

E generated = P generation × t sunlit 60

where:

- Egenerated = energy generated per orbit (Wh)

- Pgeneration = power generation (W)

- Conversion factor 60 converts minutes to hours

### Energy consumption

Energy consumed per orbit is calculated as:

E consumed = P load,sunlit × t sunlit + P load,eclipse × t eclipse 60

where:

- Econsumed = energy consumed per orbit (Wh)

- Pload,sunlit = load power during sunlit (W)

- Pload,eclipse = load power during eclipse (W)

### Net energy and power margin

Net energy per orbit is calculated as:

E net = E generated − E consumed

Power margin as a percentage is calculated as:

Margin = E net E consumed × 100 %

### Average load power

The orbit-averaged load power is calculated as:

P avg = E consumed t orbit

where the orbit period is converted from minutes to hours.

### Battery sizing

Required battery energy during eclipse is calculated as:

E battery = P load,eclipse × t eclipse 60

Required battery capacity accounting for depth of discharge is:

C battery = E battery DoD

where DoD is the depth of discharge (fraction, e.g., 0.2 for 20%).

## Instructions

### Inputs

Provide the following inputs to calculate your spacecraft power budget:

- Power generation (W): Average power available from the solar array during sunlit time. This should account for solar panel efficiency, sun angle, and any degradation factors.

- Orbit period (min): The total time for one complete orbital revolution around the central body.

- Eclipse duration (min): The time spent in eclipse (in shadow) per orbit. This must be less than or equal to the orbit period.

- Load power sunlit (W): Average total power consumption from all spacecraft loads during the sunlit portion of the orbit.

- Load power eclipse (W): Average total power consumption from all spacecraft loads during the eclipse portion of the orbit.

- Battery depth of discharge: Allowed depth of discharge as a fraction (e.g., 0.2 = 20%). Typical values range from 0.2 to 0.4 for lithium-ion batteries to maximize cycle life.

### Outputs

The tool calculates the following power budget parameters:

- Sunlit duration: Time spent in sunlight per orbit (calculated as orbit period minus eclipse duration).

- Average load power: Orbit-averaged power consumption from all loads over the entire orbital period.

- Energy generated per orbit: Total energy generated by the solar array during one orbit (sunlit period only).

- Energy consumed per orbit: Total energy consumed by all loads during one complete orbit (sunlit + eclipse periods).

- Net energy per orbit: The difference between generated and consumed energy. Positive values indicate surplus energy, negative values indicate a deficit.

- Power margin: Energy margin per orbit expressed as a percentage relative to total energy consumption.

- Required battery energy: Energy that must be supplied by the battery during eclipse to power the loads.

- Required battery capacity: Minimum battery capacity required based on the allowed depth of discharge limit.

### Notes

Important considerations when using this tool:

- The tool assumes constant power generation during sunlit periods and constant load power during sunlit and eclipse periods. Real missions may have variable power profiles.

- Power generation should account for solar panel degradation, temperature effects, and incidence angle variations throughout the orbit.

- A positive power margin indicates the system generates more energy than it consumes, allowing for battery charging and system reserves.

- Negative net energy per orbit indicates the system cannot meet power requirements and requires either increased solar array capacity or reduced load power.

- Battery depth of discharge should be selected based on battery chemistry and desired cycle life. Lower DoD values increase battery life but require larger battery capacity.

- The eclipse duration must be less than or equal to the orbit period. If eclipse duration exceeds orbit period, sunlit duration is clamped to zero.